Lab 1 - Getting Started with RISC-V (Assembly Language) in VS Code

| Name: Abdul Haseeb Khan | Student ID: 11146 |
| --- | --- |

Task 3:

Code:

main:

li x1, 5             # int a = 5;

li x2, 0             # int b = 0 + 0; (Fixed: assigned to x2 instead of read-only x0)

addi x1, x2, 32      # a = b + 32;

# int d = (a + b) - 5;

add x3, x1, x2       # temporary = a + b

addi x4, x3, -5

# int e = (((a - d) + (b - a)) + d);

sub x3, x1, x4       # temp1 = a - d

sub x6, x2, x1       # temp2 = b - a

add x7, x3, x6       # temp3 = temp1 + temp2

add x5, x7, x4       # int e = temp3 + d (e is in x5)

# e = a + b + d + e;

add x3, x1, x2       # temp4 = a + b

add x3, x3, x4       # temp4 = temp4 + d

add x5, x3, x5       # e = temp4 + e (updates e in x5)

end:

j end

Results:

Explanation:

Translating basic math into assembly. I practiced turning high-level arithmetic expressions into RISC-V instructions like add, addi, and sub using standard register assignments.

Task 4a:

- Store x10 as unsigned integer at address 0x100.

addi x15, x0, 0x100

sw x10, 0(x15)

addi x16, x0, 0x1F0

sw x11, 0(x16)

Store x11 as unsigned integer at address 0x1F0.

- Load an unsigned short integer (two bytes) from address 0x100 in x12.

lhu x12, 0(x15)

- Load a short integer from address 0x1F0 in register x13.

lh x13 , 0(x16)

- Load a singed character from address 0x1F0 in register x14.

lb x14, 0(x16)

Results:

Explanation:

Working with memory addresses and data types. I wrote code to store values into specific RAM locations and load them back using various data widths like bytes and halfwords.

Task 4b:

Code:

li x20, 0x100      # Base address of array a

li x21, 0x200      # Base address of array b

li x22, 0x300      # Base address of array c

# i = 0

lb x5, 0(x20)      # Load char a[0]

lh x6, 0(x21)      # Load short b[0]

add x7, x5, x6     # Add a[0] + b[0]

sw x7, 0(x22)      # Store to int c[0]

# i = 1

lb x5, 1(x20)      # Load char a[1]

lh x6, 2(x21)      # Load short b[1]

add x7, x5, x6     # Add a[1] + b[1]

sw x7, 4(x22)      # Store to int c[1]

# i = 2

lb x5, 2(x20)      # Load char a[2]

lh x6, 4(x21)      # Load short b[2]

add x7, x5, x6     # Add a[2] + b[2]

sw x7, 8(x22)      # Store to int c[2]

# i = 3

lb x5, 3(x20)      # Load char a[3]

lh x6, 6(x21)      # Load short b[3]

add x7, x5, x6     # Add a[3] + b[3]

sw x7, 12(x22)     # Store to int c[3]

Results:

Explanation:

Unrolling a loop to handle arrays manually. Since loops hadn't been covered yet, I manually calculated the memory offsets to load character and short array elements, add them together, and store the results into an integer array

Assessment Rubric

Lab 1: Getting Started with RISC-V (Assembly Language) n VS Code

| Name Abdul Haseeb Khan | Student ID: 11146 | Section: T-3 |
| --- | --- | --- |

Points Distribution:

|  | Task No. | LR 2 (Code) | LR 5 (Results) |
| --- | --- | --- | --- |
| In-Lab | Task 1 | - | /15 |
|  | Task 2 | - | /15 |
|  | Task 3 | /10 | /5 |
|  | Task 4a | /10 | /5 |
|  | Task 4b | /10 | /10 |
| Total Points: 100 |  | /30 | /50 |
| CLO Mapped |  | CLO 2 |  |

| Affective Domain Rubric |  | Points | CLO Mapped |
| --- | --- | --- | --- |
| AR7 | Report Submission & Git Upload | /10 & /10 | CLO 2 |

| CLO | Total Points | Points Obtained |
| --- | --- | --- |
| 2 | 100 |  |
| Total | 100 |  |